# IoT System Project

# Mini Project 1[E2]

(Smart Cabinet)

Group 10 Members

Armando Mak [223175Z]

R Sajeev [221325C]

# 1. Set up

### . Power On and Connect ESP32 Devices

- ESP32 Smartwatch: Power on the device and ensure it's connected to Wi-Fi.

- ESP32 CAM: Power on the camera module and verify the camera feed is accessible.

- ESP32 in Cabinet: Ensure the device is powered and connected to the network.

- Router: Ensure the router is connected to the pwer

### 2. Set Up and Start the Flask Web Server

- Navigate to Your Project Directory:
bash
Copy code
cd path_to_your_project_directory

- Activate Virtual Environment (if used):
bash
Copy code
source venv/bin/activate  # Linux/Mac

- .\venv\Scripts\activate   # Windows

- Start the Flask Server:
bash
Copy code
python app.py

- Check Server Availability: Open your browser and go to http://localhost:5000 to see if your server is running correctly.

### 3. Connect ESP32 Devices to the Server

- Verify Wi-Fi Connection: Ensure all your ESP32 devices are connected to the same Wi-Fi network as the server. Use Serial.print(WiFi.localIP()); on ESP32 devices to check their IP addresses.

- Update ESP32 Code (if needed): Ensure that the server IP in your ESP32 code points to the correct IP address where the Flask server is hosted.
cpp
Copy code
http.begin("http://IP-address");  // Replace with your actual server IP

### 4. Test Communication Between ESP32 Devices and Web Server

- Send Data from ESP32 Devices: Trigger actions on your ESP32 devices that send data to the Flask server (e.g., pressing a button on the cabinet).

- Monitor Server Logs: Check your server's console to verify that data is being received from the ESP32 devices.

- Access the Web Interface: Open the web interface in your browser to verify that data is being displayed correctly and that the controls work.

### 5. Network Configuration

- Static IP for ESP32 Devices: Assign static IP addresses to your ESP32 devices in your router settings to avoid reconfiguring IPs after a restart.

### 6. Secure Your Setup

- Use HTTPS: Implement HTTPS on your Flask server using SSL certificates (e.g., Let's Encrypt) for secure communication.

- Authentication: Implement basic authentication for your web interface to prevent unauthorized access.

# 2. Dashboard graphics

# 3. Setup for of sockets

# Register New ESP client

When the esp client connects to the server

ESP-Client

- The ESP client on power up will try to initialize the socket object and connect to the server

- Then it will wait for the server to request for the client for its id

|

V

Server

- Run the above code in a thread

  - In this code it will accept any socket that tries to connect to itself

  - Then check in the client to send if the socket was registered before

    - If it does not it will request id from the ESP client

  - Then start the message listener “socket_listener”

|

V

# Initialize relevant info

ESP-Client

- Client then sends “new~”

  - Which is to initialize all the other relevant info in client_to_send

  - Then wait for server to send “start”

|

V

ESP Server

- Receives the “new~”

- Checks for new client

- If it is a new client:

  - Check if the remote address is inside the client_to_send dictionary and has the “new~” message

  - Put the necessary information

    - id

      - for convenience on coding side as we can just call “cab” instead of its remote address to find its information

    - True

      - (unused)

    - STD_FB

      - To set the standard frame buffer size = 2048 bytes

  - Send the “start”

|

V

ESP-Client

- Then allow the rest of the code to proceed

# 4 & 5. Sending info using sockets

- sending can only be in utf-8 or base64

- The convention to send information from the esp client looks like:        https://www.canva.com/design/DAGJTyrvH_w/cjLpcchwf4Gd9K_r6uydag/edit?utm_content=DAGJTyrvH_w&utm_campaign=designshare&utm_medium=link2&utm_source=sharebutton

- Convention 1 is to ensure there is always space for data packets to come into the server

- Convention 2 just sends and does not declare the size:

- Just to add on, all the esp32 can do threading except the ESP32 CAM. As it will mess with its firmware, which results in error messages.

# Convention 1

This method only applies from ESP client to server

- This is to ensure no matter what the frame buffer will always be bigger

- Can be seen in the camera and the cabinet

Steps:

For the ESP client:

- For the esp-client it first has to send the size declaration

  - Then it waits for the send

On the server side:

- It will first search for the highest Frame buffer size from all the known users(default is 2048 bytes )

- Then it will receive any information at a frame buffer of the highest known frame buffer from all its users

- Receive the data at the highest frame buffer size

- Then find the id from the remote address

- See if it is a new client

- If it is not a new client, check the message and see if it is a size declaration or a message:

- Size Declaration

- If it is a size declaration:

  - Only take the size of the frame buffer and assign it to the esp-client’s frame buffer size

  - As typically the send is sent with “size-FrameBufferValue”. Where FrameBufferValue is the size of the message to be sent

  - Then send the acknowledgement “send”

For the ESP client:

- When the ESP client receives the “send” acknowledgement, then it can send the information

- Message:

- Repeat steps 2 to 4

- Find the id of the ESP client sender

# Convention 2

It can be done for

- Server to ESP client

  - The messages sent from the server to the clients are typically 3-4 letters/characters long

- ESP client to Server

- The frame buffer size will always stay the same no matter what

- Can be seen in the camera and the cabinet

  - Partly seen in smart watch

Server to ESP client

Steps:

Server

- Server first has to find the id to send

- Send the message

ESP client:

- Waits for the message to be received

  - In this case the frame buffer is 35

    - I chose 35 bytes because it should be more than enough to accommodate 3-4 letter messages

- Use the message

  - In this case if the message is lof

    - close all the lights

Server to ESP client to Server

- This is only seen in the smartwatch

  - As the server will keep requesting for the smart watch every 30 seconds

  - This is so to ensure the server will not be overloaded with all the messages

    - As it will be ready to accept the smart watch

- It is similar to Server to ESP client

Steps:

Server

- It is first initialized by a scheduler

- First check if 30s has elapsed since the last sent time or initialized time

- Server to ESP client	(same process)

  - Code used:

ESP Client

- Data is then sent

Server

- It will first search for the highest Frame buffer size from all the known users(default is 2048 bytes )

  - Since we never declared the size, the highest will be whatever the highest frame buffer from other users in that time

  - However, no matter what, the smallest size will still be 2048 bytes because the size declaration never changed as we never initialized it

- Then it will receive any information at a frame buffer of the highest known frame buffer from all its users

- Receive the data at the highest frame buffer size

- Then find the id from the remote address

- See if it is a new client

- If it is not a new client find the id of the ESP client sender